Universidade do Algarve

Faculdade de Ciências e Tecnologia

Licenciatura em Engenharia Informática

Redes de Computadores I

Laboratório 4

Wireshark – DNS Query em Python

Realizado por: Diogo Carvalho nº 90247

PL: 2

Docente: Rui Filipe Rosa Paulo

Ano Letivo: 2025/2026

# Relatório

## Codigo

DNS_query.py

import socket

import struct

import random

dnsserver = "10.10.22.228"

portnumber = 53

# transforma o domínio no formato usado pelo DNS

# exemplo: ualg.pt -> 04 ualg 02 pt 00

def encode_name(domain):

parts = domain.split(".")

name = b""

for part in parts:

name += struct.pack("!B", len(part))

name += part.encode()

name += b"\x00"

return name

# cria a mensagem da query

def build_query(domain):

transaction_id = random.randint(0, 65535)

# standard query + recursion desired

flags = 0x0100

# 1 pergunta, resto a 0

qdcount = 1

ancount = 0

nscount = 0

arcount = 0

# header DNS tem sempre 12 bytes

header = struct.pack(

"!HHHHHH",

transaction_id,

flags,

qdcount,

ancount,

nscount,

arcount

)

# question section

qname = encode_name(domain)

qtype = 1    # A

qclass = 1   # IN

question = qname + struct.pack("!HH", qtype, qclass)

return transaction_id, header + question

# lê nomes dentro da mensagem DNS

def read_name(message, offset):

labels = []

while True:

length = message[offset]

# se começar por 11, é um pointer

if (length & 0xC0) == 0xC0:

pointer = struct.unpack("!H", message[offset:offset + 2])[0]

pointer = pointer & 0x3FFF

name, _ = read_name(message, pointer)

labels.append(name)

offset += 2

break

# 0 quer dizer fim do nome

if length == 0:

offset += 1

break

offset += 1

labels.append(message[offset:offset + length].decode())

offset += length

return ".".join(labels), offset

# lê um resource record

def read_record(message, offset):

name, offset = read_name(message, offset)

rtype, rclass, ttl, rdlength = struct.unpack("!HHIH", message[offset:offset + 10])

offset += 10

rdata_start = offset

rdata = message[offset:offset + rdlength]

offset += rdlength

return {

"name": name,

"type": rtype,

"class": rclass,

"ttl": ttl,

"rdlength": rdlength,

"rdata": rdata,

"rdata_start": rdata_start

}, offset

# imprime um resource record

def print_record(record, message, number):

print("\nRegisto", number)

print("Nome:", record["name"])

print("Tipo:", record["type"])

print("Classe:", record["class"])

print("TTL:", record["ttl"])

# A -> endereço IPv4

if record["type"] == 1 and record["rdlength"] == 4:

ip = socket.inet_ntoa(record["rdata"])

print("IP:", ip)

# NS -> nome do servidor

elif record["type"] == 2:

ns_name, _ = read_name(message, record["rdata_start"])

print("NS:", ns_name)

# CNAME -> nome canónico

elif record["type"] == 5:

cname, _ = read_name(message, record["rdata_start"])

print("CNAME:", cname)

# MX -> preferência + servidor de mail

elif record["type"] == 15:

preference = struct.unpack("!H", record["rdata"][:2])[0]

mx_name, _ = read_name(message, record["rdata_start"] + 2)

print("Preference:", preference)

print("MX:", mx_name)

# outros tipos ficam em hexadecimal

else:

print("RDATA:", record["rdata"].hex())

def main():

domain = input("Domínio: ")

query_id, query = build_query(domain)

# socket UDP

client = socket.socket(socket.AF_INET, socket.SOCK_DGRAM)

client.settimeout(5)

# envia a query

client.sendto(query, (dnsserver, portnumber))

# recebe a resposta

response, address = client.recvfrom(4096)

print("\nResposta de:", address)

# header

header = struct.unpack("!HHHHHH", response[:12])

response_id = header[0]

flags = header[1]

qdcount = header[2]

ancount = header[3]

nscount = header[4]

arcount = header[5]

print("\n--- HEADER ---")

print("ID:", hex(response_id))

print("Flags:", hex(flags))

print("Questions:", qdcount)

print("Answer RRs:", ancount)

print("Authority RRs:", nscount)

print("Additional RRs:", arcount)

offset = 12

# question section

print("\n--- QUESTION SECTION ---")

for _ in range(qdcount):

qname, offset = read_name(response, offset)

qtype, qclass = struct.unpack("!HH", response[offset:offset + 4])

offset += 4

print("Nome:", qname)

print("Tipo:", qtype)

print("Classe:", qclass)

# answer section

print("\n--- ANSWER SECTION ---")

for i in range(ancount):

record, offset = read_record(response, offset)

print_record(record, response, i + 1)

# authority section

print("\n--- AUTHORITY SECTION ---")

for i in range(nscount):

record, offset = read_record(response, offset)

print_record(record, response, i + 1)

# additional section

print("\n--- ADDITIONAL SECTION ---")

for i in range(arcount):

record, offset = read_record(response, offset)

print_record(record, response, i + 1)

client.close()

if __name__ == "__main__":

main()

## Resultados

Execuçao do programa

Mensagem enviada

Resposta